# Digital Logic Design Project Report

## Title of the Project

Digital Clock Implementation Using Synchronous Counters and 7-Segment Displays

## Abstract (200-250 words)

This project presents the design and implementation of a fully functional digital clock system using synchronous counter circuits and 7-segment display interfaces in CircuitVerse simulation environment. The primary purpose is to create a real-time digital timekeeping system that displays hours, minutes, and seconds in a standard HH:MM:SS format using multiple 7-segment displays for clear visual representation.

The working principle is based on cascaded synchronous counters operating at different modulo values to achieve proper time counting. The system employs a base clock signal that drives a seconds counter (modulo-60), which generates carry pulses to drive a minutes counter (also modulo-60), which in turn provides carry signals to an hours counter (modulo-24 for 24-hour format or modulo-12 for 12-hour format). Each counter stage consists of multiple flip-flops configured to count in BCD format, enabling direct interfacing with BCD-to-7-segment decoders.

The circuit incorporates six main counter modules representing tens and units of hours, minutes, and seconds respectively. Each BCD output is connected to dedicated 7-segment display decoders that convert the 4-bit binary codes into appropriate segment control signals for the displays. The system includes proper reset functionality for time setting and synchronization.

Key outcomes include successful implementation of accurate timekeeping functionality, proper carry propagation between time units (seconds to minutes to hours), reliable BCD-to-7-segment decoding for all digits (0-9), and stable visual display representation. This digital clock demonstrates fundamental concepts in sequential logic design, timing circuits, and practical applications of counters in real-world systems.

## Circuit Diagram

Digital Clock Circuit Implementation in CircuitVerse:

The circuit shows a comprehensive digital clock system with the following structure from right to left:

### Time Display Units:

- Seconds Display: Two 7-segment displays showing tens and units of seconds (00-59)

- Minutes Display: Two 7-segment displays showing tens and units of minutes (00-59)

- Hours Display: Two 7-segment displays showing tens and units of hours (00-23 or 01-12)

### Counter Architecture:

- Seconds Counter:

  - Units counter (0-9) with modulo-10 operation

  - Tens counter (0-5) with modulo-6 operation for proper 60-second cycle

- Minutes Counter:

  - Units counter (0-9) with modulo-10 operation

  - Tens counter (0-5) with modulo-6 operation for proper 60-minute cycle

- Hours Counter:

  - Units counter (0-9) with modulo-10 operation

  - Tens counter (0-2) with modulo-3 operation for 24-hour format

### Signal Flow:

- Master Clock: Base timing signal (typically 1 Hz for real-time operation)

- Carry Chain: Overflow signals connecting seconds→minutes→hours

- Reset Logic: System-wide reset for time setting

- Display Decoders: BCD-to-7-segment conversion for each digit

### Control Elements:

- Clock Input: Primary timing source

- Reset Button: For time initialization

- Enable Signals: Individual counter stage control

## List of Apparatus

### Digital Logic Components:

- JK Flip-Flops: 24 units total

  - 4 units per BCD counter

  - 6 BCD counters (2 each for hours, minutes, seconds)

- BCD-to-7-Segment Decoders: 6 units

  - Individual decoder for each time digit display

- 7-Segment Displays: 6 units

  - 2 displays for hours (HH)

  - 2 displays for minutes (MM)

  - 2 displays for seconds (SS)

- Clock Generator: 1 unit

  - 1 Hz frequency for real-time operation

  - Adjustable for testing purposes

### Logic Gates:

- AND Gates: Multiple units

  - Carry generation logic

  - Reset logic implementation

  - Counter overflow detection

- OR Gates: Multiple units

  - Signal combination

  - Reset distribution

- NOT Gates: Multiple units

  - Signal inversion

  - Logic complementation

- NAND Gates: Multiple units

  - Alternative logic implementation

  - Reset logic

### Control Circuits:

- Reset Circuit: 1 unit

  - Manual time setting capability

  - System initialization

- Modulo Logic Circuits: 6 units

  - Modulo-10 for units digits

  - Modulo-6 for tens of seconds and minutes

  - Modulo-3 for tens of hours (24-hour format)

### Simulation Environment:

- CircuitVerse Digital Logic Simulator

  - Web-based simulation platform

  - Real-time circuit simulation

  - Interactive testing environment

- Timing Analysis Tools

  - Built-in timing diagram generation

  - Signal monitoring capabilities

  - Performance verification

### Project Files and Documentation:

- Circuit Modules:

  - Main circuit file

  - Sub-circuit modules (Synchronous BCD, 7-seg-BCD-final, etc.)

- Test Benches:

  - Functional verification setups

  - Timing validation circuits

### Technical Specifications:

- Display Format: HH:MM:SS (6-digit time display)

- Counting Range: 00:00:00 to 23:59:59 (24-hour format)

- Update Rate: 1 second intervals

- Reset Capability: Manual and automatic reset functions

- Power Requirements: Digital logic levels (simulated environment)