4th Year  B.S.(Honours) ,Test Examination-2024

Course Title: Nuclear and Particle Physics, Code: PHA 406

Time:3 Hours,     Full marks-70

| 1. | (a) | Define microscopic and macroscopic cross section of neuron and establish a relation between them. | 4 |
| --- | --- | --- | --- |
|  | (b) (c) | Write down Breit-Wigner formula and discuss the significance of its three parts. What is fission cross-section? Explain of it. | 4 3 |
|  | (d) | What do you mean by neutron activation? Discuss its importance. | 3 |
| 2. | (a) (b) | What is meant by fuel cycle. Sketch all steps of a nuclear fuel cycle and briefly discuss each step. Write the characteristics of the fuels used in a nuclear reactor. | 12 2 |
| 3. | (a) | Sate Fick’s law. Write down the assumptions to establish the law. | 3 |
|  | (b) | Derive the steady state equation of continuity. | 4 |
|  | (c) (d) | Show that transport mean path equals three times of diffusion coefficient. Determine the transport mean free path for thermal neutron in 1m3 hydrogen gas at standard temperature and pressure. Scattering cross section of neutron is 38 burn. | 4 3 |
| 4. | (a) | Describe prompt neurons and delayed neutrons. Show that delayed neutrons are absent then the number of neutrons produced in a reactor increases exponentially. | 7 |
|  | (b) | obtain the maximum fractional energy loss of a neutron in collision with a moderator nucleus | 4 |
|  | (c) | Calculate the number of collision to reduce the energy of neutron from 2.0 MeV Calculate to 0.025eV with 9Be nucleus. | 3 |
| 5. | (a) (b) | What do you mean by super critical, critical and sub critical of a nuclear reactor. Deduce critical equation for a large reactor Type equation here. | 4 4 |
|  | (c) | What is non- leakage probability? Deduce an expression of critical size of a open reactor. | 6 |
| 6. | (b) | What is reactor period? Derive an expression for reactor period. | 5 |
|  | (b) | What are differences between fuel rods and control rods. | 3 |
|  | (c) | Write the names of two substances used as moderator, coolant and reflector. | 3 |
| 7. | (a) | What are the thermal problems in reactor design? | 4 |
|  | (b) | Discuss the fundamental principles of heat removal from a reactor. | 5 |
|  | (b) | Write the names of two substances used as moderator, coolant and reflector. | 3 |
|  | (c) | Describe how the reprocessing of irradiated fuel in used of a nuclear reactor. | 5 |
| 8. | (a) | Describe the structure and working principle of a swimming pool nuclear reactor and mention its limitations | 16 |
|  | (b) | Write the differences between a power reactor and a research reactor. |  |
|  |  | Discuss the different types of radiation sources in a nuclear reactor. |  |
|  |  | Write the fundamental safety principles of nuclear. |  |
|  |  | Write down the uses of nuclear reactor. |  |
|  |  | Describe the cause of generating heat in reactor core. |  |
|  |  | . Diffusion theory is not valid near fuel rod, control rod and at the edge of the reactor-explainDefine: Fuel Pellet, Fuel rod and Fuel Assembly. Write the fundamental safety principles of nuclear. |  |